CPM Verification Sign-Off Document

Project: Configurable Packet Modifier (CPM) Verification

Version: 1.1

Author: Assaf Afriat

Date: 2026-02-15

Status: SIGNED OFF

# 1. Executive Summary

The CPM (Configurable Packet Modifier) UVM verification environment has been completed and all closure criteria have been met. The design (RTL v1.1) has been verified to function correctly across all 4 operation modes, with comprehensive functional coverage achieved. All previously reported DUT bugs have been fixed and verified.

# 2. Closure Criteria Checklist

## 2.1 Mandatory Requirements

| # | Criterion | Status | Evidence |
| --- | --- | --- | --- |
| 1 | All tests pass (no runtime hangs) | PASS | All tests complete successfully |
| 2 | Scoreboard reports 0 mismatches | PASS | 485 matched, 0 mismatched |
| 3 | All assertions pass (no failures) | PASS | 4/4 assertions (100%) - 0 violations |
| 4 | Functional coverage targets achieved | PASS | See coverage section |
| 5 | RAL reset sequence passes cleanly | PASS | CpmRalResetTest PASS |
| 6 | End-of-test invariants checked | PASS | Counter invariant verified via RAL |
| 7 | UVM_ERROR count = 0 | PASS | 0 errors in simulation |
| 8 | Callback mechanism demonstrated | PASS | CpmPacketStatsCb tracks 505 packets |

2.2 UVM Mechanisms Demonstrated

| Mechanism | Requirement | Status | Implementation |
| --- | --- | --- | --- |
| RAL | Mandatory | Implemented | CpmRegModel, CpmRegAdapter, uvm_reg_predictor |
| Virtual Sequence | Mandatory | Implemented | CpmTopVirtualSeq with full 8-step flow |
| Factory Override | Mandatory | Implemented | CpmBaseTrafficSeq → CpmCoverageTrafficSeq |
| Callbacks | Mandatory | Implemented | CpmPacketStatsCb with real statistics tracking |
| Functional Coverage | Mandatory | Implemented | CpmPacketCoverage, CpmRegCoverage |
| SVA Assertions | Mandatory | Implemented | 4 assertions + 4 cover properties in CpmStreamIf.sv |

3. Test Results

3.1 Test Execution Summary

| Test | Execution Time | Result | Errors | Warnings |
| --- | --- | --- | --- | --- |
| CpmSmokeTest | ~1 sec | PASS | 0 | 0 |
| CpmMainTest | ~2 sec | PASS | 0 | 1 (expected) |
| CpmRalResetTest | ~1 sec | PASS | 0 | 0 |

3.2 Scoreboard Results (CpmMainTest - RTL v1.1)

| Metric | Value |
| --- | --- |
| Packets Input | 505 |
| Packets Matched | 485 |
| Packets Dropped (intentional) | 20 |
| Packets Mismatched | 0 |
| Match Rate | 100% |

3.3 Callback Statistics (NEW in v1.1)

| Metric | Value |
| --- | --- |
| Total Packets Tracked | 505 |
| Opcodes Exercised | 16/16 (100%) |
| Payload Range | 0x0000 - 0xFFFF |

4. Coverage Results

4.1 Functional Coverage (Mandatory Targets)

| Coverpoint | Target | Achieved | Status |
| --- | --- | --- | --- |
| MODE | 100% | 100.00% | MET |
| OPCODE | 90% | 100.00% | MET |
| MODE × OPCODE | 80% | 100.00% | MET |
| Drop bin | Hit once | 50% | MET |
| Stall bin | Hit once | 50% | MET |

4.2 Covergroup Summary

| Covergroup | Coverage | Status |
| --- | --- | --- |
| cg_packet | 100.00% | MET |
| cg_register | 75.00% | MET |

4.3 Code Coverage (DUT)

| Metric | Coverage |
| --- | --- |
| Statements | 100.00% |
| Branches | 95.55% |
| Expressions | 100.00% |
| Conditions | 88.88% |
| Toggles | 58.62% |
| Total DUT | 88.61% |

4.4 Assertion Coverage

| Metric | Result |
| --- | --- |
| Total Assertions | 4 |
| Passed | 4 |
| Failed | 0 |
| Cover Properties | 4 |
| Pass Rate | 100% |

5. Feature Verification

5.1 Operation Modes

| Mode | Transformation | Latency | Verified |
| --- | --- | --- | --- |
| PASS (0) | payload unchanged | 0 cycles | Yes |
| XOR (1) | payload ^ mask | 1 cycle | Yes |
| ADD (2) | payload + constant | 2 cycles | Yes |
| ROT (3) | rotate left by 4 | 1 cycle | Yes |

5.2 Registers

| Register | Address | Reset Value | Verified |
| --- | --- | --- | --- |
| CTRL | 0x00 | 0x00000000 | Yes |
| MODE | 0x04 | 0x00000000 | Yes |
| PARAMS | 0x08 | 0x00000000 | Yes |
| DROP_CFG | 0x0C | 0x00000000 | Yes |
| STATUS | 0x10 | 0x00000000 | Yes |
| COUNT_IN | 0x14 | 0x00000000 | Yes |
| COUNT_OUT | 0x18 | 0x00000000 | Yes |
| DROPPED_COUNT | 0x1C | 0x00000000 | Yes |

5.3 Protocol Properties

| Property | Status |
| --- | --- |
| Input stability under stall | Verified (0 violations) |
| Output stability under stall | Verified (0 violations - RTL v1.1 fixed) |
| Bounded liveness | Verified (0 violations - drop awareness) |
| Counter invariant | Verified (RTL v1.1 fixed) |

6. Bug Summary

6.1 DUT Bugs (RTL v1.1)

| Bug ID | Description | Severity | Status |
| --- | --- | --- | --- |
| DUT-001 | COUNT_OUT over-count | High | FIXED |
| DUT-002 | Output stability violation | High | FIXED |
| DUT-003 | ROT_AMT not configurable | Medium | Resolved (spec clarified) |

6.2 Testbench Bugs

| Bug ID | Description | Severity | Status |
| --- | --- | --- | --- |
| TB-001 | VIF config_db collision | Critical | Closed |
| TB-002 | Reset wait race | High | Closed |
| TB-003 | Zero-time loop | High | Closed |
| TB-004 | Ref model config stale | Medium | Closed |
| TB-005 | FIFO ordering assumption | Medium | Closed |
| TB-006 | RAL model not built | Critical | Closed |

Total Open Bugs: 0

7. Spec Compliance (v1.1)

7.1 Resolved Items

| Item | Status | Notes |
| --- | --- | --- |
| ROT_AMT value | Resolved | Spec v1.1 confirms fixed at 4 bits |
| Counter invariant timing | Resolved | STATUS.BUSY defines when invariant holds |
| SOFT_RST scope | Resolved | "clears counters and internal state" |

7.2 Resolved Spec Ambiguities (v1.0 → v1.1)

| Item | Original Concern | Resolution |
| --- | --- | --- |
| Ordering contradiction | Different latencies vs "no reordering" | Resolved - RTL maintains FIFO ordering regardless of mode latency |
| Counter overflow | Undefined 32-bit wrap behavior | Resolved - RTL uses standard wrap-around (implementation-defined per spec) |
| ENABLE deassertion | In-flight packet handling | Resolved - RTL flushes pipeline when ENABLE deasserted (implementation-defined) |
| in_ready when disabled | Value when ENABLE=0 | Resolved - RTL correctly deasserts in_ready when ENABLE=0 |

All spec ambiguities from v1.0 have been resolved.

8. Deliverables Checklist

| Deliverable | Status |
| --- | --- |
| UVM Testbench Source Code | Complete |
| SVA Assertions | Complete |
| RAL Code | Complete |
| Verification Plan | Complete |
| Coverage Report | Complete |
| Test Results | Complete |
| Bug Tracker | Complete |
| Documentation | Complete |
| Callback Implementation | Complete |

9. Sign-Off

9.1 Verification Engineer

| Name | Role | Date | Signature |
| --- | --- | --- | --- |
| Assaf Afriat | Verification Engineer | 2026-02-07 | Approved |

9.2 Approval

This document confirms that the CPM verification environment meets all mandatory closure criteria as defined in the CPM Final Project Verification Requirements and Deliverables document.

Verification Status: COMPLETE

RTL Version: v1.1 (all bugs fixed)

Spec Version: v1.1

Document End

Last Updated: 2026-02-15